# Support Flow Tensor Field for Fast Build-Orientation Candidate Generation in Support-Requiring Additive Manufacturing

Abstract

Build orientation strongly controls support volume in additive manufacturing, but exhaustive orientation sweeps remain costly. The proposed method targets support-requiring additive-manufacturing processes in which build orientation changes the amount of removable support material; it is not intended for inherently support-free processes. This paper proposes the Support Flow Tensor Field (SFTF), a fast candidate generator that ranks promising build directions before local verification by a support-volume grid or slicer. For each sampled direction, SFTF casts rays from overhang faces to actual receiver faces and forms a direction-dependent asymmetric flow tensor. The build-plate support term is further interpreted as a virtual ground receiver, making the dominant predictive term a unified Rayleigh cost. On five benchmark meshes with a 1∘ tomography grid, verifying the ±10∘ neighborhoods of the top-20 SFTF candidates uses 3.57% of the grid on average and obtains a mean support-volume ratio of 1.13 with maximum 1.58. Budget-matched uniform and random local-window controls are weaker, with mean/maximum ratios of 1.64/2.79 and 2.62/5.44. The paper also introduces the happy-method, an automatic hard-case escalation rule based on local-verification confidence signals. On the five stored meshes it reduces the worst ratio from 8.52 to 1.06 while using 2.99% of the full grid on average; on a five-group cached audit, it reduces the maximum ratio from 12.44 to 1.62 at a mean budget of 4.96%. These results show that SFTF is best viewed not as a replacement for precise support-volume optimization, but as a low-cost warm start that concentrates expensive verification on a small number of likely basins.

Keywords: 3D printing, support structure, asymmetric flow tensor, direction dependence, ground-node unification

# Introduction

## Build Orientation and the Support Structure Problem

In additive manufacturing, and especially in the most widely used FDM (fused deposition modeling) method, molten filament is stacked layer by layer [1]. To print a downward overhang steeper than the critical angle (critical angle, θc), a support structure (support structure, SS) is needed to hold it up from below. The support structure is a sacrificial structure that is removed after printing, but it consumes additional filament and print time, and degrades surface quality through removal marks and staircasing. Therefore the support volume is a key factor governing the cost and quality of FDM printing. Accordingly, this study targets support-requiring additive-manufacturing processes in which the selected build orientation changes the amount of removable support material, rather than inherently support-free processes.

Even for the same shape, the distribution of overhangs changes depending on the pose in which it is placed on the build plate, so the build orientation directly affects the following factors.

- Support volume

- Print time

- Material usage

- Surface quality

- Print stability

Among these, finding the build orientation that minimizes the support volume is the optimal-orientation search problem that this study addresses.

## Related Work

Build orientation optimization and support structure reduction are long-studied topics in additive manufacturing design, and prior approaches can be broadly divided into (i) orientation optimization, which finds a build orientation with a small support volume, (ii) support structure design, which optimizes the shape of the support structure for a given orientation, and (iii) design-stage approaches, which reduce the support requirement itself by changing the geometry or topology of the part.

In orientation optimization, Das et al. voxelized the mesh and selected a build orientation that jointly considers the support volume and part error [2], and Wang et al. used a layered depth–normal image representation and GPU acceleration to quickly process the geometric operations of complex polyhedra, laying the foundation for voxel-based additive manufacturing computation [3]. Such explicit, voxel-based evaluation gives the support volume of a single direction accurately, but when searching over directions exhaustively it incurs a large fixed cost independent of shape complexity (shown quantitatively in §4.3).

On the support structure design side, research that minimizes support volume with lattice or cellular structures is active. Vaissier et al. pruned lattice support structures with a genetic algorithm to reduce material consumption [4], and Jiang et al. experimentally identified the printable critical overhang angle in extrusion-based additive manufacturing to reduce unnecessary support [5]. As attempts to reduce the support requirement at the design stage, Mirzendehdel and Suresh proposed topology optimization that includes support structures as a constraint [6], and Langelaar presented a unified formulation that simultaneously optimizes part topology, support placement, and build orientation [7]. Meanwhile, Dai et al. demonstrated an alternative paradigm that achieves support-free printing using multi-axis motion [8].

In particular, a line of work that expresses the directional dependence of overhang directly as anisotropy or a tensor has developed within the topology-optimization community, and this is the family most closely related to the tensor formulation of the present study. Gaynor et al. introduced a maximum overhang constraint into SIMP topology optimization, forcing each layer to be supported by the solid below it within the self-supporting angle [9]. Garcke et al. penalized overhanging boundaries through anisotropic interface energy (an anisotropic interface energy of Wulff-shape type) in a phase-field topology optimization, so that self-supportable boundary-normal directions are preferred [10]. Shin et al. targeted multi-axis additive manufacturing and simultaneously optimized a deposition-direction-dependent material stiffness anisotropy tensor D(ϕ)=T(ϕ) D0 T(ϕ)T, an overhang penalty, and the stage-wise build orientations [11]. On the industrial side, Smith and Hollenbeck patented a method that selects the build orientation of complex metal parts with many orthogonal and internal faces—where support is hard to remove—so as to minimize support structure and surface distortion [12].

This study also shares with the anisotropy family above the idea of capturing the directionality of overhang with a tensor, but its position is complementary. The tensors in those works represent the physical anisotropy of material stiffness (Shin) or interface energy (Garcke) inside a PDE-constrained design optimization that changes the part topology/density, with the build direction either fixed (Gaynor, Garcke) or co-optimized as a design variable (Shin). By contrast, the Support Flow Tensor Field of this study is a purely geometric support-flow descriptor evaluated per candidate direction on a fixed part, used to rank build orientations at low cost without redesigning the part. That is, without changing the shape of the support structure or the part topology, and assuming standard FDM single-direction printing, it focuses on the build orientation, but rather than replacing the expensive exhaustive evaluation, it aims for a candidate generator and warm-start that, in front of it, narrows down promising directions at low cost and passes them to an accurate verifier or the local verification of a commercial slicer. The patent of Smith and Hollenbeck [12] is close to this study in goal (selecting the build orientation of a fixed shape) but is an industrial method whose procedure is not disclosed, whereas this study provides a transparent tensor cost and a verifier warm-start. The next section first summarizes the implicit-estimation family that this study directly extends (shadow-volume-based MSST).

## Optimal Orientation Search: Explicit/Implicit Methods and MSST

The optimal build orientation can be obtained explicitly or implicitly. Explicit methods actually slice each candidate direction to generate G-code and directly compute the support volume. They are accurate but expensive, so prior works, even after voxelizing the mesh and accelerating G-code generation with GPUs, generally remained at single-direction evaluation due to the computational burden and could not be applied to exhaustive optimal-orientation search.

Implicit methods indirectly estimate the support volume without complex G-code generation. Representatively, Ezair et al. observed that the volume of the support structure is similar to the shadow volume produced by a virtual sun [13]. This is intuitive but holds only when the critical angle is 0∘, i.e., when every overhang requires support. Previous work [14] generalized this idea to an arbitrary critical angle and developed it into “modified support structure tomography (MSST).”

MSST divides the virtual ray into many rays and treats each ray’s intersection with the surface as a “pixel.” Within a “pixel slot” that sorts pixels at the same (x,y) location by their Z coordinate, it classifies upward-facing faces (α) and downward-facing faces (β), and estimates the support volume (Vss) by subtracting the object volume (Vo) and the non-overhang volume (Vnv) from the top-cover volume (Vtc) in the form Vss=Vtc−Vo−Vnv. This approach reproduced almost the same support volume as a commercial slicer on several standard meshes such as the Stanford Bunny.

## GPU-MSST and Remaining Limitations

The only weakness of MSST was speed. A single-direction computation of the 65k-triangle Bunny takes about 1 second, and sweeping yaw–pitch at 1∘ intervals requires 360×360=129,600 single-direction computations, so a single optimal-orientation search takes 1–2 minutes on a CPU. Recent GPU-MSST work reconstructs MSST with GPU-friendly data structures (slot-based pixel data structure, triangle-split voxelization) [15], shortening the same exhaustive search to about 10 seconds and reporting on average 11.2× acceleration relative to a high-end CPU and up to 62× relative to a low-cost CPU. In this paper we use this GPU-based MSST implementation as an accurate support-volume verifier and refer to it as TOMO_INT3.

However, GPU-MSST also essentially searches the entire yaw–pitch grid exhaustively. That is, the acceleration made single-direction evaluation fast; it did not reduce the number of directions to be evaluated (129,600). As a result, the search cost has a large fixed cost almost independent of shape complexity (in §4.3 of the main text, even a cube with only 12 faces takes about 4 seconds for a 3∘ TOMO_INT3 sweep), and is excessive for simple shapes or fast preliminary review. Indeed, the prior work itself started from the observation that “only a limited portion between the α faces and β faces contributes to the final support structure,” and presented, as future work, a tensorial approach (tentatively named shadow tensor) that reduces the evaluation targets. This study is precisely the follow-up work in that direction.

## This Study: Support Flow Tensor Field

The goal of this study is not to replace the expensive exhaustive search, but to build a candidate generator that quickly suggests promising build orientations in front of it, serving simultaneously as a slicer/TOMO warm-start. By evaluating a low-cost support cost for each candidate direction on the sphere, it narrows down only a small number of promising orientations, and feeds these into the local verification of TOMO_INT3 or a commercial slicer, thereby reducing the overall search cost.

For this, a tensor that summarizes the shape is needed. The simplest candidate is the symmetric tensor that collects the distribution of face normals with area weighting, M=i​AimimiT, which in this paper we call the “Support Tensor” as a comparison baseline. However, as shown in §2.1, this symmetric tensor only captures the distribution of normals and cannot explain where the support structure actually flows from and to. In particular, it does not reflect concave regions, internal occlusion, the height to the build plate, or the visibility changes that depend on the build orientation.

Therefore, for direction n∈S2 this study directly evaluates an asymmetric Support Flow Tensor Field F(n) (§2.2). F(n) accumulates, via ray casting, the face-to-face support “flow” from an overhang face toward the actual receiver face beneath it, and is treated not as a simple eigenvector problem but as a per-direction cost evaluation problem. Also, as the core unifying perspective of this study, the build-plate support cost is formulated not as a separate scalar penalty but as the Rayleigh contribution obtained when adding a “virtual ground node whose normal is the build direction” to the flow tensor. In this way, face-to-face self-support and build-plate support are unified into a single flow tensor (unified Rayleigh cost R=R+B, §2.4.1), and in the quantitative analysis (§4.2.4) we confirm that this unified term drives the predictive power.

In summary, the contributions of this study are as follows.

- We define an asymmetric Support Flow Tensor Field F(n) for each build orientation, and directly evaluate face-to-face internal visibility via ray casting.

- We show that the build-plate support cost is exactly equivalent to the Rayleigh contribution obtained by adding a virtual ground node to the flow tensor (Eq. [eq:Rtilde]), unifying face-to-face self-support and build-plate support into a single tensor.

- Through correlation analysis, ablation, and cross-validation on a stored TOMO_INT3 grid, we identify the source of the predictive power and quantitatively verify the advantage over the PCA and Support Tensor eigenvector baselines.

- We introduce the happy-method: an automatic hard-case escalation protocol that uses local TOMO confidence signals to expand the verification budget only when the ordinary SFTF top-K search is unreliable.

- On a difficulty-controlled five-group shape set (including large meshes with ∼106 faces), we compare Python SFTF and C++ SFTF with the TOMO_INT3/TOMO_CUDA 3∘ exhaustive sweep, presenting computation-time reductions of 1.4–34× and 10–5,160× relative to TOMO_INT3 respectively, as well as a per-direction cost-structure interpretation that a single vss cannot provide, and we analyze the change in TOMO optimal orientation and support volume across critical angles θc∈{0,30,45,60,90}∘.

# Method

## The Baseline (Symmetric) Support Tensor and Its Limitations

#### Terminology (important).

Since two tensors with similar names appear in this paper, we first distinguish them. The Support Tensor is a symmetric tensor that collects the distribution of face normals with area weighting; it is not newly proposed by this study but is a classical construction used as a comparison baseline. It belongs to the same family as the orientation tensor (or fabric tensor) of fiber/particle mechanics or the structure tensor of image processing, i.e., the “second moment of the normal vectors.”  By contrast, what this study newly proposes is the direction-dependent asymmetric tensor of the next section (§2.2), the Support Flow Tensor Field F(n). In short, the “Support Tensor” is an existing baseline, and the “Support Flow Tensor Field” is the contribution of this study. Meanwhile, the anisotropic interface-energy or stiffness tensors used to handle overhang in topology optimization [10, 11] form yet a third category distinct from both: they are not descriptors of the directional cost of a fixed shape but physical anisotropy models within a design optimization that redesigns the part (§1.2).

#### Definition and Properties.

The baseline (symmetric) Support Tensor is defined as

M=i​AimimiT

Here Ai is the area of triangular face i, and mi is its outward normal. Since M is a symmetric positive semidefinite matrix,  an eigenvector  interpretation is possible. That is, the eigenvectors of M tell us which direction the shape’s normals point on average (the principal direction).

#### Limitations.

However, M only captures the distribution of normals and not the process by which the support structure is actually generated. Specifically, M does not include the following information.

- Which faces are overhangs in a particular direction

- Whether there is another actually supportable face beneath an overhang face

- The height or distance to the support target

- The region that the build plate must directly support

- The asymmetric face-to-face support flow

Therefore, the eigenvectors of the Support Tensor can explain the statistical directionality of the shape, but they do not always coincide with the optimal direction of the support volume that the slicer computes. This limitation is the starting point for introducing a per-direction asymmetric flow tensor in the next section.

## Per-Direction Support Flow Tensor Field

We take the build orientation as a unit vector

n∈S2,  ∥n∥=1

Letting the centroid of face i be ci, the outward normal mi, and the area Ai, the overhang coefficient at direction n is defined as

Oi(n)=max(0,−mi⋅n)

Let j be the first valid face encountered by casting a ray in direction −n from overhang face i. The height difference is then

hij(n)=(ci−cj)⋅n

and only the case hij>0 is recognized as a support flow.

The face-to-face influence is set as

wij(n)=Oi(n)1+αhij(n)

The current implementation uses

α=1D

where D is the bounding box diagonal of the input mesh.

Therefore the Support Flow Tensor for direction n is computed as

F(n)=(i,j)∈R(n)​wij(n)AiAjmimjT

Here R(n) is the set of valid face-to-face support relationships obtained by ray casting.  Since F(n) is a sum of mimjT, it is in general not a symmetric matrix.

## Ray Casting and Valid Support Relationships

For each candidate direction n, ray casting every overhang face would be computationally expensive. Therefore, the current implementation selects source faces by deterministic stratified sampling  in proportion to the overhang contribution

AiOi(n)

The number of rays is set according to the number of faces Nf as

Kray=clip(0.02Nf,500,3000)

To avoid self-intersection, the ray origin is slightly shifted as

ri(0)=ci−εn

and the ray direction is −n. The current implementation uses

ε=10−6D

A valid receiver face j must satisfy the following conditions.

- j≠i.

- The ray distance is sufficiently positive.

- hij(n)>10−5D.

- The receiver normal faces the build direction sufficiently.

The last condition is implemented as

mj⋅n>η

and currently uses η=0.05. This condition removes vertical or flipped faces that are hard to regard as actual receiver faces even if the ray reached them.

## Build Plate Term

An overhang face for which ray casting cannot find a support face below must eventually be held up by the build plate or by a separate support structure. The current implementation computes this term as a scalar, but as shown in §2.4.1, it is exactly equal to the Rayleigh contribution obtained when adding the build plate to the flow tensor as a virtual receiver.

The build-plate height reference for direction n is defined as

zplate(n)=minv∈Vv⋅n

The height of face i to the build plate is

hiP(n)=ci⋅n−zplate(n)

Letting B(n) be the set of source faces with no valid face-to-face hit, the build-plate support penalty is computed as

B(n)=i∈B(n)​Oi(n)Ai1+αhiP(n)

This term increases the cost the more a tall overhang directly requires support all the way to the build plate.

### Virtual Ground Node Interpretation of the Build Plate

B(n) looks like a scalar penalty added outside the tensor, but it is in fact equivalent to the Rayleigh contribution obtained when adding the build plate as a virtual receiver (ground node) to the face-to-face flow tensor F(n). In the face-to-face flow term mimjT, the receiver normal mj faces upward with mj⋅n>η>0, and the support-face normal of the build plate is exactly the build direction n (m⋅n=1, i.e., the limiting receiver that faces most upward). Therefore, treating a face i∈B(n) supported by the plate as a “flow pair with a virtual node whose normal is n,” we define the augmented tensor as follows.

F(n)=(i,j)∈R(n)​wij(n)AiAj mimjT⏟face-to-face flow+i∈B(n)​Ai1+αhiP(n) minT⏟build-plate flow (virtual node).

Since mi⋅n=−Oi(n), the Rayleigh contribution of the plate term is

nT​12(minT+nmiT)​n=(mi⋅n) (n⋅n)=−Oi(n)

and multiplying by the coefficient Ai(1+αhiP) and summing, the amount that the plate term contributes to −nTFsn is exactly B(n). Therefore the Rayleigh support cost of the augmented tensor holds exactly as

R(n)=max​0, −nTFs(n) n=R(n)+B(n)

This equivalence was numerically confirmed as max|R−(R+B)|<10−12 over the entire candidate pool of the five stored meshes (about 1,292 candidates each) (§4.2.4).

The significance of this reformulation is twofold. First, the build-plate cost that accounts for most of the predictive power (§4.2.3) turns out to be not an ad-hoc scalar but a term of the flow tensor, justifying the “tensor” formulation. Second, the build plate is not a separate mechanism but is unified as “the limit of face-to-face self-support (a fully horizontal receiver)”— every overhang flows to a receiver, and that receiver is either another face or the build plate. The asymmetry of the height convention is intentional. The face-to-face weight uses attenuation 1/(1+αh) (distant self-support is weakened), while the plate term uses amplification (1+αh) (a tall support column is costly). That this sign difference is not arbitrary is verified by quantitative comparison with other conventions in §4.2.4.

The two flow terms of the unified tensor. (a) When overhang i is supported by an actual face j, the receiver normal is mj⋅n∈(η,1) and the tensor term is mimjT. (b) When there is no face to support it, it is supported by the build plate, “a limiting receiver whose normal is exactly n” (m⋅n=1), and the term becomes minT. The build-plate cost B(n) is exactly equal to the Rayleigh contribution of this ground term (Eq. [eq:Rtilde]).

## Per-Direction Objective Function

Geometric meaning of the overhang coefficient Oi(n)=max(0,−mi⋅n). The possible face-normal directions are shown on a unit circle with respect to the build direction n. The upper semicircle (mi⋅n>0) consists of upward faces with Oi=0, and the lower shaded region is the cone of half-angle θc around −n, where faces with −mi⋅n>cosθc are overhangs that actually require support. Oi grows the more the normal aligns with −n (the further down it goes).

To use the asymmetric tensor F(n) as a directional cost, we define the symmetric component as

Fs(n)=F(n)+F(n)T2

The initial approach used nTFs(n)n directly, but experiments showed that under strong support flow the sign of the value could become negative, flipping the alignment direction. Therefore the current implementation converts the Rayleigh term  into a non-negative support cost.

R(n)=max0,−nTFs(n)n

The face-to-face support volume term is set as

P(n)=(i,j)∈R(n)​Oi(n)1+αhij(n)AiAj1+αhij(n)

Here the height attenuation factor 1/(1+αhij) included in the face-to-face influence wij and the outer (1+αhij) cancel each other, so P(n) ultimately simplifies to

P(n)=(i,j)∈R(n)​Oi(n)AiAj

That is, P(n) represents the area-weighted overhang amount itself over the valid hits, and the height dependence is intentionally removed. This is a choice to keep the height attenuation in the tensor F(n) so as to weaken the contribution of distant face-to-face flows, while interpreting the face-to-face support volume term P(n) as the pure self-supportable area amount.

Using the augmented tensor of §2.4.1, the Rayleigh term R and the build-plate term B merge into a single unified Rayleigh cost R=R+B (Eq. [eq:Rtilde]). Therefore the final objective function can be written as the sum of the unified tensor term and the face-to-face support volume

J(n)=λRR(n)+λPP(n)+λBB(n)=R(n)⏟R+B+P(n)

with (λR=λP=λB=1), and from the unified-tensor perspective it is interpreted as “the Rayleigh support cost R of the augmented flow tensor plus the face-to-face support area amount P.” As shown in §4.2.4, R itself is a better predictor than J, because the summed P acts as a negative signal. In the initial implementation, from the singular values  σ1≥σ2≥σ3≥0 of F(n) we added a nuclear score  S(n)=k​σk(n) and a σ1(n) term with weight 0.05, but in the ablation of §4.2.3 these two terms were confirmed not to change the candidate selection at all, so they were excluded from the objective function. The singular values are kept only as auxiliary features for post-processing rank tuning.  (§4.2.5). The optimal build orientation is defined as

n*=argminn∈S2J(n)

## Spherical Sampling and Local Re-search

Using only three eigenvectors does not sufficiently reflect the asymmetry and direction dependence of the Support Flow Tensor Field. Therefore the current implementation uses the following two-stage search that follows a coarse-to-fine (local re-search) strategy [reference needed] of evaluating a sparse global sample and then densely resampling only around promising candidates.

- Generate 512 coarse directions by the Fibonacci sphere [22, 23] method.

- Compute J(n) for each direction.

- Select the top 12 candidates, but remove directions that are too close to each other.

- Resample around each top candidate with 2∘, 4∘, 6∘, 8∘ cones.

- For each cone angle, evaluate 16 azimuths.

- Combine the coarse and refined candidates and re-sort.

- Finally return the top 3 directions that are at least 3∘ apart from each other.

This approach is faster than densely evaluating the entire sphere, while still being able to reflect sensitivity to small angular changes around the optimal direction.

Two-stage direction search. After evaluating coarse candidates (gray, 512) that sparsely cover the sphere via the Fibonacci sphere, the regions around the top candidates (stars) are densely resampled with 2∘–8∘ cones (blue dots) to reflect sensitivity near the optimal direction. Local precision is obtained without densely evaluating the entire sphere.

## Computational Acceleration and Final Candidate Rescoring

Since the per-direction Support Flow Tensor Field casts many rays for each candidate direction, a direct implementation has its computational bottleneck mostly in ray–triangle intersection [24]. The initial implementation obtained all hits for each ray and then sorted by distance to find the first valid receiver face. However, since for most rays the nearest hit is immediately the valid hit, computing all hits every time creates unnecessary cost.

The current implementation first performs a first-hit query with

multiple_hits=False

Only when this first hit violates the self-face, too-short distance, negative height difference, or receiver normal condition is the existing multiple-hit query performed as a fallback for that ray. That is, over the entire ray set Q, only

Qfallback={q∈Q∣firstHit(q) is invalid}

is searched additionally. This strategy maintains the existing exact valid-hit selection rule, while reducing the number of intersection candidates and the sorting cost for most rays.

Also, the face normals, areas, centroids, vertex arrays, bounding box diagonal D, α=1/D, ray offset ε, and height tolerance, which used to be recomputed for each candidate direction, were computed only once per mesh and cached. For this, the current implementation shares

Dmesh={mi,Ai,ci,V,D,α,ε,εh}

across the entire Support Flow evaluation.

In profiling on the Bunny 1k mesh, the existing direction evaluation time was about 15.7 seconds, but after applying first-hit fallback and mesh surface data caching it decreased to about 1.5 seconds. On the same mesh, the selected top direction, score, and hit count matched those of the existing multiple-hit method. Therefore this optimization can be viewed as an implementation-level acceleration that reduces only the computational amount without changing the meaning of the algorithm.

### Final Candidate Rescoring Based on TOMO Results

The original objective function J(n) was effective for candidate generation and local re-search, but it was not always well aligned with the actual support volume vss of TOMO_INT3 when choosing the final top 3 candidates. In particular, a direction with a small B(n) term is not always a direction with a small actual support volume, and a candidate with too small a face-to-face flow amount and hit count may not actually form sufficient self-support relationships.

Therefore, the current implementation, after building the coarse and refined candidate sets, rescores them in the final sorting stage using rank-normalized features within the candidate set. Rank normalization, which converts feature values into intra-set ranks instead of using them directly, is a known technique for robustly combining features with different scales [reference needed]. For each candidate nk, the following feature vector is set.

xk=Jk, Rk, Pk, Bk, Hk, Sk, σ1,kT,

where Hk is the valid hit count. For each feature x(l), the intra-set rank normalization is defined as

ρk(l)=rank(xk(l))N−1  0≤ρk(l)≤1

The rank of a candidate with a small value is close to 0, and that of a candidate with a large value is close to 1.

The final tuning score is set as

T(nk)=wJρk(J)+wRρk(R)+wPρk(P)+wBρk(B)+wHρk(H)+wSρk(S)+w1ρk(σ1)

The current experimental implementation used, based on per-candidate comparison with the stored TOMO_INT3 grid,

wJ&=0,&wR&=0.731,&wP&=−2.3897,wB&=2.1447,&wH&=2.3109,&wS&=−2.2922,&w1&=1.6363

These values are not constants directly derived from physical laws, but an empirical calibration of the current implementation’s feature scale to the TOMO_INT3 validation set. Therefore, in this paper we interpret them not as a final theoretical formula but as one example of calibrating the Support Flow Tensor Field with TOMO or slicer feedback. The candidate generation itself is performed based on the existing J(n), and the rank tuning is applied only at the final candidate selection stage. This is a choice to avoid overly aggressive re-search direction changes, and to maintain the structural meaning of the existing Support Flow objective function.

## Current Implementation Pseudocode

Normalize n Compute Oi(n)=max(0,−mi⋅n) for all faces Select Kray source faces by stratified sampling with weight AiOi(n) Initialize F←0 , P←0 Cast ray from ci−εn along −n Find the first valid receiver face j hij←(ci−cj)⋅n;wij←Oi(n)/(1+αhij) F←F+wijAiAjmimjT P←P+wijAiAj(1+αhij) hiP←ci⋅n−zplate(n) F←F+Ai(1+αhiP) minT Fs←(F+FT)/2 R←max(0,−nTFsn) J(n)←R+P (singular values σ1,σ2,σ3 are computed from the face-to-face subtensor F only as auxiliary features for rank tuning) Generate refined directions around the best coarse candidates under J(n) Evaluate J(n) and feature vector x(n) for all coarse and refined candidates Rank-normalize each feature inside the candidate set Compute final tuning score T(n) Sort candidates by T(n) with angular non-maximum suppression Verify the top-Kbase=10 candidates by local TOMO/slicer windows Compute η=Vssloc/Vbbox, g=Vssseed/Vssloc, and the window-edge flag Activate happy-method: add top-30 SFTF windows and optional Pareto candidates Re-verify the union of all local windows Activate severe-boundary tier: add top-100 SFTF windows Re-verify the union of all local windows Return the verified direction with the smallest measured Vssloc

In the implementation, instead of accumulating the build-plate term into the tensor, it is summed separately as a scalar B to equivalently compute R=R+B (the equivalence of §2.4.1, numerically confirmed within 10−12). The above pseudocode makes explicit the unified-tensor form that is mathematically the same as that scalar accumulation. The face-to-face subtensor F is the part of F excluding the ground-node term, and is used only for computing the singular value features.

# Evaluation Protocol and Datasets

## Datasets and Experimental Environment

The dataset consists of five groups that deliberately separate and control the two axes of shape difficulty and mesh scale. A–C set the scale in the ∼101–105-face range and vary the difficulty axis from basic shapes to freeform organic shapes, revealing the fixed-cost floor of the exhaustive sweep (a cost almost independent of mesh complexity). D–E fix the difficulty as real manufacturing parts while pushing the scale axis from 256 faces to 1.04×106 faces, revealing the adaptive cost by which the SFTF candidate-evaluation cost increases adaptively to mesh complexity. Therefore A–E is not a single size ramp, and the partial overlap of the face-count ranges of groups C and D is the result of a design that separates and controls the two axes, not an ordering error.

- A (basic shapes, 5; lower difficulty): cube, sphere, cylinder, cone, torus (face count ∼101–103).

- B (simple functional parts, 5; mid difficulty): u_bracket, hook, c_clamp, pipe_elbow, hollow_box.

- C (organic/freeform benchmarks, 5; upper difficulty): standard scan/sculpture meshes such as Bunny (69,662 faces), manikin, dragon (∼105 faces), happy, lucy. The Stanford grid meshes used in §4.2.6 are exactly this group.

- D (Thingi10K set 1, 10; lower scale): real manufacturing parts with 256–88,106 faces [25].

- E (Thingi10K set 2, 10; upper scale): large high-resolution meshes with about 5.1×105–1.04×106 faces.

All timing experiments were run on a Windows 11 Education 64-bit workstation with an AMD Ryzen 9 9950X3D CPU (16 cores, 32 logical threads), 125 GB RAM, and an NVIDIA GeForce RTX 5080 GPU (16 GB, driver 595.79). The Python implementation used Python 3.12.13 with NumPy 1.26.4, SciPy 1.17.1, Trimesh 4.4.0, Matplotlib 3.11.0, and scikit-learn 1.9.0. TOMO_INT3 and TOMO_CUDA were called through Windows DLL interfaces, and the SFTF_C++ implementation was built as a Windows x64 C++ DLL using the Visual Studio 2022/MSVC toolchain with OpenMP parallelization.

## Comparison Targets and Evaluation Protocol

We first define the comparison targets and evaluation procedure used in the quantitative analyses up to §4.3. The comparison targets are as follows.

- PCA-based direction

- Existing Support Tensor eigenvector direction

- Initial eigenvector-based concave direction

- This study’s Support Flow Tensor Field direction

- TOMO_INT3 or commercial-slicer-based vss optimal direction

For each mesh, we generate a vss contour at 1∘ intervals under a critical angle of 60∘, and evaluate how close the vss value at our method’s top candidate directions is to the global optimum. Even if the direction coordinates do not match exactly, if the vss values are similar we regard them as optimal orientations of the same level. Also, considering the case where the global optimum appears scattered across multiple basins, we compare angularly separated top-k basin representatives rather than simple top-k grid points. The candidate basins proposed by SFTF are re-input to TOMO_INT3 as a single initial orientation with angle step 0∘ to directly measure the Mss, and based on that measured value we mark the measured optimal and measured worst basins.

# Results and Discussion

## Comparison with Stored TOMO_INT3 Results

To verify the current implementation, we used the *_tomo_int3_vss_grid_1deg_60deg.npz files stored in Experimental/etc. Each file contains the TOMO_INT3 support volume vss computed on a yaw–pitch 1∘ grid, and the global minimum direction of that grid. In the comparison process, instead of recomputing the TOMO grid, we mapped the top directions proposed by the current Support Flow Tensor Field onto the TOMO grid, and read the vss value at that location to compare with the global optimum.

The TOMO yaw–pitch corresponding to an SFTF direction n was chosen as the grid point that best satisfies, for TOMO’s rotation matrix R(yaw,pitch,0),

R(yaw,pitch,0)n≈ez

Since a direction vector and its sign reversal can represent the same build axis, both n and −n were evaluated and the smaller vss value was used as the signed result.

Comparison of the stored TOMO_INT3 vss grid with the current SFTF candidates. Ratio is the ratio of the SFTF candidate’s vss to the TOMO global optimal vss.

| Mesh | TOMO best | SFTF score1 | Ratio | SFTF best-of-3 | Ratio |
| --- | --- | --- | --- | --- | --- |
| Bunny 69k | 3201.39 | 4665.85 | 1.46 | 4665.85 | 1.46 |
| manikin | 4657.72 | 8675.96 | 1.86 | 8675.96 | 1.86 |
| dragon 100k | 24446.98 | 45804.41 | 1.87 | 39547.57 | 1.62 |
| happy 50k | 2112.47 | 25467.94 | 12.06 | 23727.49 | 11.23 |
| lucy 50k | 5036.14 | 14192.68 | 2.82 | 14192.68 | 2.82 |

As Table 1 shows, SFTF with rank tuning applied selected candidates closer to the TOMO optimum than the existing raw score for Bunny, manikin, and dragon. In particular, Bunny’s signed score1 ratio improved from about 3.03 to 1.46, manikin from about 3.99 to 1.86, and dragon’s best-of-3 signed ratio from about 2.01 to 1.62. By contrast, the improvement was limited for happy and lucy.

However, the rank tuning results in Table 1 are in-sample values measured on the very mesh set on which the weights were calibrated. Therefore these numbers are not generalization performance but a fit to the calibration set, and whether the weights are overfit and the performance on unseen meshes are evaluated separately by the cross-validation of §4.2.5. Also, how much each feature predicts the actual vss, and whether the asymmetric tensor term actually contributes to the result, are analyzed in §4.2.3.

### Marking the Optimal Orientation Based on Top-k Basins Rather Than Top-k Points

In the yaw–pitch contour of TOMO_INT3, the global optimum may appear not as a single isolated point, but at almost the same value in several mutually separated basins. For example, in one mesh both the vicinity of (yaw,pitch)=(69∘,130∘) and the vicinity of (265∘,49∘) corresponded to the global optimum or a near-global optimum. In this case, simply marking the top 3 grid points with small vss values would repeatedly select only neighboring grid points within the same basin, failing to convey to the user the information that “other good orientation groups also exist.”

Top-k points vs. Top-k basins. When there are several mutually separated low-cost basins (blue regions) on the yaw–pitch cost landscape, simply picking the top points with small vss (red stars) repeatedly selects only neighboring points within one basin. With angular non-maximum suppression (NMS), keeping only candidates that are at least θbasin away from the already-chosen representatives (blue circles), different near-optimal orientation groups are presented as representatives.

Therefore, the current visualization and post-processing mark the top basins (top-k basins) rather than the top points (top-k points). After converting a yaw–pitch point (ψ,ϕ) to a build direction n(ψ,ϕ), the distance between two orientations is defined as

d(na,nb)=cos−1(na⋅nb)

Here we use na⋅nb rather than |na⋅nb|. This is because, in the yaw–pitch orientation of TOMO_INT3, n and −n can represent different build orientations. Scanning the sorted candidate list in order, a candidate whose angular difference from an already-selected basin representative satisfies

d(n,nr)<θbasin

is considered the same basin and removed. The current implementation used θbasin=12∘.

For the TOMO_INT3 contour itself, after sorting the vss grid in ascending or descending order, we apply the angular non-maximum suppression  above to select 3 optimal-basin representatives and 3 worst-basin representatives respectively. This approach shows, when the global optimum exists in multiple places, different optimal orientation groups instead of redundant grid points of the same basin.

The same basin perspective is applied to the SFTF candidates as well. First, we sort candidates with low and high final tuning score T(n) respectively, but we remove same-basin duplicates to select low-score basin candidates and high-score basin candidates respectively. The current implementation keeps up to 40 basin candidates per direction. Then we put each candidate as the initial yaw–pitch orientation of TOMO_INT3, set the angle step to 0, and compute only a single orientation. That is, for candidate qk=(yawk,pitchk) we directly measure

Mssk=TOMO_INT3(qk;Δθ=0)

Finally, we re-sort the candidate basins by this Mssk value, and mark the 3 with small measured Mss as measured optimal basins and the 3 with large measured Mss as measured worst basins.

This procedure does not interpret the order of the SFTF scores directly as the final optimal/worst. Instead, SFTF plays the role of quickly proposing a broad candidate basin, and TOMO_INT3 is used as an accurate single-orientation verifier for a small number of candidates. Therefore the user can confirm not only a single optimal orientation, but several orientation groups that are mutually separated yet have a small actual support mass. This is especially useful when, in build orientation optimization, “which selectable optimal orientation groups exist” is more important than the coordinates of the rank-1 itself.

In an additional analysis, we mapped all coarse/refined candidates onto the TOMO grid to check the lowest vss within the candidate pool. With the default 512 coarse directions, the candidate pool of happy 50k itself did not sufficiently include the vicinity of the TOMO optimum. Increasing the number of coarse directions to 2048 lowered the lowest ratio within the candidate pool of happy 50k to about 1.91. This means that the failure of happy 50k is not merely a score tuning problem, but also includes limitations of the direction-sampling resolution and the candidate-generation method. However, in the 2048-candidate pool there was difficulty in stably selecting a good candidate with the current features alone. Good candidates were generally located in an intermediate region where the build-plate penalty is low while the pair flow and hit count are not too low. Therefore, in the future, rather than a simple linear rank score, a feature that directly reflects the face-to-face self-support coverage or a TOMO-style support-volume proxy is needed.

The main improvement factors confirmed in this experiment are as follows.

- The sign of the overhang normal and the build direction sign were directly reflected.

- Face-to-face internal visibility was evaluated via ray casting.

- A valid receiver face condition was imposed to consider only actually supportable faces.

- Overhangs that must be directly supported all the way to the build plate were handled as a separate penalty.

- Instead of 3 eigenaxes, spherical sampling and local re-search were used.

- In the final candidate selection, rank-normalized feature tuning based on comparison with the TOMO_INT3 grid was applied.

- In the final visualization, angularly separated top-k basin representatives were marked rather than top-k points, providing the user with different near-optimal orientation groups.

At the same time, these results suggest that the current SFTF is useful not so much as a complete replacement of the TOMO_INT3 optimum, but as a candidate generator or warm-start method for reducing the expensive yaw–pitch global search. In particular, after proposing a few top candidates, locally verifying only their vicinity with TOMO or a slicer can greatly reduce the overall search cost.

## Accuracy and Feature Analysis

In this section we quantitatively analyze the validity of the proposed method from several perspectives. All analyses reuse the stored 1∘ TOMO_INT3 vss grid and do not require GPU recomputation, and are reproducible with the public analysis scripts. Unless otherwise noted, the ranking-only best-of-3 ratio is the ratio, relative to the TOMO global optimum, of the smallest signed vss among the top 3 candidates (3∘ angular NMS) when mapped onto the TOMO grid, and smaller is better. We separately report a local-verification budget curve, where the top-K SFTF candidates define ±10∘ yaw–pitch windows and the best TOMO grid value inside the union of those windows is measured.

### Local TOMO Verification Budget Curve

The top-3 ranking-only metric is intentionally conservative: it asks whether the learned score alone chooses the final orientations. However, the operational use of SFTF is to reduce a global TOMO or slicer search to a small number of local checks. Therefore we evaluated a budget curve over K, using the saved exhaustive 1∘ TOMO_INT3 grids as a deterministic proxy for local TOMO re-verification. For each mesh, the SFTF candidates were sorted, angularly separated, mapped to signed TOMO yaw–pitch coordinates, and the best vss inside the cumulative ±10∘ windows was recorded.

Budget curve of top-K local TOMO verification around SFTF candidates. The budget is the mean fraction of the 1∘ TOMO grid covered by the union of local windows.

| Top-K | Mean ratio | Median ratio | Max ratio | Mean grid budget | Ratio ≤2.0 |
| --- | --- | --- | --- | --- | --- |
| 3 | 2.59 | 1.15 | 8.52 | 0.87% | 4/5 |
| 10 | 2.52 | 1.00 | 8.52 | 2.12% | 4/5 |
| 20 | 1.13 | 1.00 | 1.58 | 3.57% | 5/5 |
| 30 | 1.01 | 1.00 | 1.06 | 5.43% | 5/5 |

To verify that this improvement is not merely a consequence of spending ∼3–5% of the grid, we also ran budget-matched non-adaptive controls (Table 3). For each mesh, the unique grid-cell count used by SFTF top-20 was used as a hard cap. The uniform-axis control uses the same 512 Fibonacci-sphere axes in a purely geometric farthest-first order, and the random control reports the per-mesh median over 500 random permutations of the same axes.

Budget-matched local TOMO controls at the per-mesh SFTF top-20 grid-cell budget. SFTF improves both the mean and worst-case ratio, showing that the warm-start value is the placement of local windows, not only the budget size.

| Policy | Mean | Median | Max | Budget | ≤2.0​ |
| --- | --- | --- | --- | --- | --- |
| SFTF top-20 local | 1.13 | 1.00 | 1.58 | 3.57% | 5/5 |
| Uniform-axis matched | 1.64 | 1.38 | 2.79 | 3.57% | 4/5 |
| Random matched (median) | 2.62 | 1.75 | 5.44 | 3.57% | 3/5 |

This curve changes the interpretation of the earlier best-of-3 value. At K=3, the mean remains high because happy 50k is a single hard case with a local ratio of 8.52. At K=20, all five meshes fall below a ratio of 2.0, and the worst case is reduced to 1.58 while checking only 3.57% of the grid on average. At K=30, the mean ratio is 1.01, indicating that SFTF already places useful basins near the TOMO optimum when it is allowed to pass a modest verification budget downstream. Thus, the appropriate claim is not that SFTF replaces TOMO with one ranked top-3 list, but that it converts the exhaustive yaw–pitch search into a small, high-yield local verification problem.

### Happy-Method: Automatic Hard-Case Escalation

The budget curve also shows that applying the largest budget uniformly is unnecessary: only happy 50k requires a substantially wider local verification budget, while the other meshes are already resolved by top-10 local verification. Therefore we define the happy-method as an adaptive escalation protocol rather than a manually selected exception. The detector uses only quantities available after the ordinary local verification step and does not use the global TOMO optimum. Let Vssloc be the best support volume found by top-10 local verification, Vssseed the best value among the top-10 candidate centers, and Vbbox the bounding-box volume of the mesh. We use

triggerhappy&=η≥0.03∧edge=1 ∨ g≤1.5∨edge=1∧g≥3.0,η&=VsslocVbbox,  g=VssseedVssloc,

where edge=1 means that the best local cell lies within 1∘ of the boundary of at least one ±10∘ verification window. Intuitively, a high normalized support volume indicates that the current local result is still expensive for the size of the part, while a boundary minimum or a small seed-to-local improvement indicates that the local search has not found a stable basin. The second branch, edge=1 and g≥3.0, captures a different failure mode: the local verifier greatly improves over the seed but the best cell still lies at the search boundary, suggesting that a useful basin continues outside the current window. Because η is an absolute normalized-support threshold, parts whose true minimal support is inherently large may trigger escalation even near their optimum when the boundary or gain condition is also met; this only spends extra verification budget and never degrades the selected orientation, but it helps explain the relatively high trigger rate (7/16) in the A–E audit.

When the high-support branch of Eq. [eq:happy-trigger] is true, we keep all already verified candidates and add the wider top-30 SFTF local windows. When the severe-boundary branch is true, we use a top-100 tier. Because the final orientation is selected by the verified TOMO value over a superset of candidates, the escalation can only increase the verification budget; it does not replace a previously verified better candidate. We also evaluated an optional variant that adds Pareto-filtered candidates, but on the five stored meshes the top-30 SFTF escalation alone already resolves the hard case and the severe-boundary branch is not activated.

Automatic happy-method escalation from top-10 local TOMO verification. The detector of Eq. [eq:happy-trigger] selected only happy 50k for escalation.

| Policy | Expanded | Mean | Max | Budget | ≤2.0​ |
| --- | --- | --- | --- | --- | --- |
| Top-10 local | 0/5 | 2.52 | 8.52 | 2.12% | 4/5 |
| Happy-method | 1/5 | 1.01 | 1.06 | 2.99% | 5/5 |
| Happy-method+Pareto | 1/5 | 1.01 | 1.06 | 3.18% | 5/5 |
| All-mesh top-30 | 5/5 | 1.01 | 1.06 | 5.43% | 5/5 |

Thus the hard case does not require a human to identify happy 50k in advance, nor does it require all meshes to pay the top-30 verification cost. The adaptive policy recovers the same maximum ratio as the all-mesh top-30 budget, but reduces the mean grid budget from 5.43% to 2.99% by expanding only the automatically detected hard case. We note that the thresholds in Eq. [eq:happy-trigger] were set so that the detector fires only on happy among the five stored meshes, so Table 4 is in-sample for the detector. Accordingly, Table 4 should be read as a calibrated demonstration of the escalation mechanism, not as independent evidence of detector generalization. The cached A–E audit (Table 5) is the held-out check; it is, however, limited to the 16 ratio-valid records out of 35 audited meshes, since the remaining 19 have nonpositive TOMO best at θc=60∘ and leave the ratio undefined.

After adding this escalation to the implementation pseudocode, we re-audited the five-group cache using the saved 3∘ TOMO_INT3 grids and the precomputed SFTF candidate CSVs. This audit does not rerun the TOMO DLL or regenerate candidates; it checks whether the final pipeline in Algorithm 1 changes the A–E validation conclusions. Records with nonpositive TOMO best values were excluded from ratio statistics because the ratio is undefined, leaving 16 ratio-valid records. The severe-boundary branch was needed for one D-group record whose top-10 result had a large ratio but low normalized support.

Cached five-group audit of the tiered happy-method on ratio-valid 3∘, θc=60∘ TOMO_INT3 records. The top-10 baseline over all ratio-valid records has mean ratio 2.07, maximum 12.44, mean budget 2.45%, and 13/16 records below ratio 2.0.

| Group | n​ | Expanded | Mean | Max | Budget | ≤2.0​ |
| --- | --- | --- | --- | --- | --- | --- |
| A | 1 | 1 | 1.07 | 1.07 | 7.25% | 1/1 |
| B | 1 | 0 | 1.00 | 1.00 | 2.74% | 1/1 |
| C | 5 | 1 | 1.09 | 1.23 | 3.08% | 5/5 |
| D | 5 | 3 | 1.26 | 1.61 | 7.20% | 5/5 |
| E | 4 | 2 | 1.10 | 1.21 | 4.48% | 4/4 |
| All | 16 | 7 | 1.14 | 1.62 | 4.96% | 16/16 |

The A–E audit supports the intended interpretation of the happy-method: the pipeline pays more only on automatically detected low-confidence records, while the mean verified ratio remains close to one and every ratio-valid record falls below 2.0. At the same time, the D-group severe-boundary case shows why the escalation must be tiered rather than a single fixed top-30 rule. The budget-matched controls in Table 3 further show that the SFTF-seeded windows are substantially more useful than non-adaptive uniform-axis or random windows at the same grid budget.

### Feature Correlation and Objective Function Ablation

First, how much each feature predicts the actual vss was evaluated by the feature–vss Spearman rank correlation  over the entire candidate pool (Table 6).

Spearman rank correlation between features and TOMO vss over the entire candidate pool. The larger the positive value, the more valid the feature is as a cost.

| Feature | Bunny | manikin | dragon | happy | lucy | Average |
| --- | --- | --- | --- | --- | --- | --- |
| J=R+P+B​ | 0.46 | 0.26 | 0.80 | 0.27 | 0.57 | 0.47 |
| R (Rayleigh) | 0.40 | −0.33​ | −0.57​ | −0.03​ | 0.14 | −0.08​ |
| P (pair flow) | −0.04​ | −0.38​ | −0.72​ | 0.00 | −0.59​ | −0.34​ |
| B (build-plate) | 0.50 | 0.66 | 0.85 | 0.29 | 0.62 | 0.58 |
| H (hit count) | 0.24 | −0.53​ | −0.73​ | −0.00​ | −0.71​ | −0.35​ |
| S (nuclear) | −0.02​ | −0.36​ | −0.70​ | −0.05​ | −0.26​ | −0.28​ |
| σ1​ | −0.03​ | −0.34​ | −0.66​ | −0.07​ | 0.07 | −0.21​ |

Only the build-plate term B(n) has a consistent positive correlation across all meshes (0.29–0.85, average 0.58), even higher than the predictive power of the entire objective function J (average 0.47). By contrast, P, S, σ1 derived from the asymmetric tensor and the hit count H have negative average correlations and flip sign from mesh to mesh. That is, these terms do not generalize as cost terms with a fixed sign.

To re-confirm this at the objective-function level, we sorted candidates by several objective-function variants and then measured the best-of-3 ratio (Table 7).

Best-of-3 ratio of the objective-function ablation (lower is better). The proposed objective function J=R+P+B and the old objective function including nuclear/σ1 are completely identical per mesh.

| Ranking variant | Bunny | manikin | dragon | happy | lucy | Average |
| --- | --- | --- | --- | --- | --- | --- |
| T (rank tuned, in-sample) | 1.46 | 1.86 | 1.62 | 11.23 | 2.82 | 3.80 |
| J=R+P+B (proposed) | 2.47 | 3.99 | 2.01 | 11.15 | 2.82 | 4.49 |
| R+P+B+0.05S+0.05σ1 (old objective) | 2.47 | 3.99 | 2.01 | 11.15 | 2.82 | 4.49 |
| B alone | 2.57 | 2.60 | 1.94 | 16.04 | 2.82 | 5.19 |
| 0.05S+0.05σ1 (tensor alone) | 5.82 | 5.38 | 6.57 | 11.86 | 5.62 | 7.05 |
| P alone | 5.82 | 4.92 | 6.27 | 12.07 | 7.51 | 7.32 |

Crucially, the results of the proposed objective function J=R+P+B and the old objective function including the nuclear/σ1 terms are identical across all meshes. This means that the nuclear/σ1 terms with weight 0.05 essentially do not contribute to the final candidate selection, and is the basis for excluding these terms from the objective function in this revision. Also, the variants using only tensor-derived terms (0.05S+0.05σ1, P alone) have the worst ratios. That is, the predictive power of our method originates not from the asymmetric tensor structure but from the build-plate penalty and the internal visibility evaluation, and the tensor terms play only an auxiliary role.

### Reformulation into a Unified Tensor: Rayleigh Equivalence of the Build-Plate Term

The ablation in §4.2.3 showed that the build-plate term B takes almost full charge of the predictive power and that the Rayleigh term R derived from the tensor flips sign from mesh to mesh (Table 6). However, according to the virtual ground node interpretation of §2.4.1, B itself is a Rayleigh contribution of the augmented flow tensor, and the unified Rayleigh cost is exactly expressed as R=R+B (Eq. [eq:Rtilde]). This section verifies whether that unified term is actually a better predictor.

First, we confirmed the equivalence numerically. Computing the Rayleigh cost of the augmented tensor F(n) directly over the entire candidate pool of the five stored meshes, it was identical within floating-point error, with |R−(R+B)|<10−12 for all candidates.

Table 8 compares the Spearman correlation of the unified term R with the single terms. With R alone the average is −0.08 (unstable sign), but R=R+B with the ground node added has an average of 0.60, the best among single features, even higher than the current objective function J=R+P+B (average 0.47). The reason J is lower than R is that the face-to-face support volume P (average correlation −0.34, Table 6) acts as a negative signal and lowers the predictive power when summed.

Spearman correlation of the unified Rayleigh term R=R+B and single terms with TOMO vss (higher is better). R was computed directly from the augmented tensor and agrees with R+B within 10−12.

| Mesh | R​ | B​ | R=R+B​ | J=R+P+B​ |
| --- | --- | --- | --- | --- |
| Bunny | 0.40 | 0.50 | 0.52 | 0.46 |
| manikin | −0.33​ | 0.66 | 0.65 | 0.26 |
| dragon | −0.57​ | 0.85 | 0.89 | 0.80 |
| happy | −0.03​ | 0.29 | 0.31 | 0.27 |
| lucy | 0.14 | 0.62 | 0.62 | 0.57 |
| Average | −0.08​ | 0.58 | 0.60 | 0.47 |

This was also confirmed by the best-of-3 ratio (Table 9). Sorting candidates by the unified term alone (i.e., removing P), the average ratio over the 4 non-hard-case meshes improves from J’s 2.82 to R’s 2.50. happy is reported separately: at the default 512 directions its candidate points evaluate far from the optimum under the ranking-only point metric (oracle 4.35), and on this mesh the unified term is rather worse. We stress that this reflects the narrow, sharp optimum basin of happy under point evaluation, not a failure of candidate generation: as shown in §4.2.2, a usable seed lies within the top-30 pool (within a ±10∘ window of the optimum), and local windowed verification recovers a ratio of about 1.06.

Best-of-3 ratio when sorting candidates by J=R+P+B and the unified term R=R+B (lower is better). On the average excluding happy (a narrow-basin hard case under the ranking-only point metric, resolved by local verification in §4.2.2), R outperforms J.

| Mesh | J=R+P+B​ | R=R+B​ |
| --- | --- | --- |
| Bunny | 2.47 | 2.57 |
| manikin | 3.99 | 2.89 |
| dragon | 2.01 | 1.70 |
| lucy | 2.82 | 2.82 |
| Average (excl. happy) | 2.82 | 2.50 |

In summary, the unified tensor reformulation (i) formulates the build-plate cost as a term of the flow tensor, justifying the “tensor” representation, and (ii) shows that the single-tensor Rayleigh cost R with the negative signal P removed is consistently superior to the existing J.

#### Verification of the Height Convention.

The ground term uses a height convention opposite in sign to the face-to-face term (amplification instead of attenuation). To show that this asymmetry is not arbitrary, we compared four plate conventions. The current amplification Bamp=∑OiAi(1+αh), the height-free Bflat=∑OiAi, the attenuation Batt=∑OiAi/(1+αh) symmetric with the face-to-face term, and Buni=∑Oi2Ai/(1+αh) that fully follows the face-to-face weight law. The average Spearman correlations were 0.58, 0.51, 0.43, 0.47 respectively, so the current amplification convention is the highest and most consistent, and the attenuation convention Batt matched symmetrically with the face-to-face term was the lowest (negative correlation on happy). That is, the sign asymmetry “distant self-support is weakened, tall build-plate support is strengthened” is physically correct and empirically supported. However, the fully unified convention Buni greatly improves best-of-3 on slender shapes such as manikin (e.g., manikin 2.89→1.37), leaving room for a shape-adaptive convention in the future.

### Cross-Validation of Rank Tuning

Since the rank tuning weights in Table 1 were calibrated together on the reported meshes, to evaluate generalization performance we performed Leave-One-Mesh-Out (LOMO) cross-validation.  Setting each mesh as hold-out and refitting the weights on the remaining meshes (ridge regression  on the intra-mesh rank-normalized features, with the target being the intra-mesh rank-normalized vss), we measured the best-of-3 ratio of the hold-out mesh. Excluding happy, whose narrow optimum basin makes the ranking-only point metric unrepresentative (it is instead resolved by local windowed verification, §4.2.2; see also the oracle below), we evaluated on the 4 meshes whose ranking-only point oracle is already near the TOMO optimum.

LOMO cross-validation best-of-3 ratio of rank tuning (lower is better). The in-sample column is the calibration-set fit value reported in Table 1. The oracle is the best direction existing within the candidate pool, representing the upper bound of candidate generation.

| Mesh | oracle | untuned J | in-sample (reported) | LOMO |
| --- | --- | --- | --- | --- |
| Bunny | 1.46 | 2.47 | 1.46 | 2.20 |
| manikin | 1.13 | 3.99 | 1.86 | 2.62 |
| dragon | 1.29 | 2.01 | 1.62 | 1.86 |
| lucy | 1.02 | 2.82 | 2.82 | 2.82 |
| Average | 1.22 | 2.82 | 1.94 | 2.37 |

The reported in-sample average 1.94 is not reproduced in honest cross-validation, and the ranking-only hold-out average is 2.37, about 0.43 (about 22%) higher. This difference is the optimistic bias of the weight calibration. Also, among the 7 weights, only R (≈0.7) and B (≈1.0) are reproduced stably across folds, while the reported large P=−2.39, H=2.31, S=−2.29, σ1=1.64 are not reproduced in any fold. In fact, the LOMO average ratio of reduced models using only stable features is B alone 2.48, R+B 2.46, R+P+B 2.44, not much different from the full 7-feature model (2.37), and all are superior to untuned J (2.82). Therefore, instead of 7 calibrated weights, tuning a few features centered on R, B (and P if needed) is simpler yet more robust. When happy is included, the reduced models are also more stable than the full 7-feature model, which confirms that feature tuning alone should not be used to explain all failure cases. For ordinary meshes, the gap between the ranking-only hold-out value and the oracle mainly reflects a scoring/ranking problem. For happy-like cases, however, the issue is better handled as a candidate-coverage and local-verification-budget problem (see §4.2.1 and §4.2.7).

To test whether the same conclusion survives beyond the five stored Stanford-like meshes, we expanded the validation set using the five-group TOMO_INT3 cache. The expanded set contains 103 usable mesh–critical-angle records from 34 meshes after excluding records with nonpositive TOMO best values or missing candidate CSVs. We used mesh-held-out nested cross-validation so that different critical-angle conditions of the same mesh did not leak into both training and validation.

Expanded G5 mesh-held-out nested CV over 103 usable mesh–critical-angle records. The median and tail statistics are reported because a small number of high-angle hard cases dominate the mean.

| Metric | Mean | Median | P80 | P90 | Ratio ≤2.0 |
| --- | --- | --- | --- | --- | --- |
| Oracle in pool | 1.13 | 1.01 | 1.08 | 1.17 | 100/103 |
| Untuned J | 4.66 | 1.41 | 2.30 | 4.29 | 75/103 |
| Raw R+B | 3.96 | 1.34 | 2.45 | 4.44 | 77/103 |
| B nested | 3.97 | 1.34 | 2.21 | 3.59 | 80/103 |
| R+B nested | 4.15 | 1.42 | 2.29 | 3.81 | 78/103 |
| R+P+B nested | 4.22 | 1.41 | 2.51 | 4.21 | 75/103 |
| Full 7 nested | 4.75 | 1.48 | 2.74 | 5.01 | 72/103 |

The expanded validation changes the reporting emphasis. The median selected ratio is 1.34, and 80/103 cases fall below a ratio of 2.0, but the mean remains about 4.0 because a few D/E high-angle cases produce very large ranking errors. Therefore, in the expanded setting, median/P80 and explicit hard-case accounting are more informative than a single mean ratio.

### Unified Comparison with Existing Methods

Finally, under the same ranking-only best-of-3 ratio criterion we compared PCA directions, Support Tensor eigenvector directions, and SFTF (untuned and LOMO cross-validation tuned) (Table 12). PCA uses the principal axes of the area-weighted face-centroid covariance, and Support Tensor uses the eigenvectors of M=i​AimimiT, taking the best value over the 3 axes of each method (including ±). The SFTF-tuned column is not an in-sample value but the LOMO value of §4.2.5.

Unified comparison of existing methods and SFTF (best-of-3 signed vss ratio, lower is better). SFTF-tuned is the cross-validated generalization estimate.

| Mesh | PCA | Support Tensor | SFTF untuned | SFTF tuned (LOMO) | oracle |
| --- | --- | --- | --- | --- | --- |
| Bunny | 2.82 | 3.71 | 2.47 | 2.20 | 1.46 |
| manikin | 2.65 | 2.50 | 3.99 | 2.62 | 1.13 |
| dragon | 2.69 | 2.33 | 2.01 | 1.86 | 1.29 |
| lucy | 6.37 | 8.67 | 2.82 | 2.82 | 1.02 |
| Average | 3.63 | 4.30 | 2.82 | 2.37 | 1.22 |

Cross-validated SFTF in the conservative ranking-only setting (average 2.37) is about 35% better than PCA (3.63) and about 45% better than Support Tensor eigenvectors (4.30), and even untuned J (2.82) is better than the two existing methods. In particular, the Support Tensor eigenvectors are worse than even PCA (8.67 on lucy), which quantitatively supports this study’s starting assumption that the statistical principal axes of the normal distribution alone cannot explain the direction-dependent support volume. This comparison shows that even when discarding the reported in-sample numbers and using the conservative cross-validation numbers, SFTF’s advantage over existing methods is maintained. The local-verification curve in Table 2 should be read as the operational version of this result: the ranking-only value is a lower-level diagnostic of the score, while the deployed pipeline uses SFTF to allocate a small TOMO verification budget.

### Additional Robustness Analysis: Critical Angle, Sampling Density, Dimensional Normalization

#### Critical Angle Matching.

SFTF’s overhang definition Oi(n)=max(0,−mi⋅n) includes all downward faces regardless of the critical angle, but TOMO_INT3 regards as support-needing only the faces with −mi⋅n>cosθc among downward faces (mi⋅n<0). As a result of applying the same gate to the SFTF overhang to match the definitions, under the θc=60∘ setting (same as TOMO) the LOMO average best-of-3 ratio worsened from 2.37 to 2.82, and at θc=45∘ it was 2.42, similar to the baseline. That is, matching the critical angle gate does not provide a consistent improvement, which is interpreted as the rank tuning already absorbing the difference in the overhang definition. Therefore the default implementation does not apply the gate.

#### Sampling Density and Point-Oracle Limit.

Increasing the number of coarse Fibonacci sphere directions, we measured the best value within the candidate pool (oracle ratio) (Table 13).

Oracle (best within candidate pool) ratio by the number of coarse directions. Meshes other than happy already include a near-optimal direction at 512.

| # coarse directions | Bunny | manikin | dragon | happy | lucy |
| --- | --- | --- | --- | --- | --- |
| 512 | 1.46 | 1.13 | 1.29 | 4.35 | 1.02 |
| 1024 | 1.50 | 1.11 | 1.33 | 3.94 | 1.02 |
| 2048 | 1.61 | 1.06 | 1.28 | 1.91 | 1.04 |
| 4096 | 1.62 | 1.13 | 1.27 | 1.91 | 1.30 |

happy’s oracle ratio is 4.35 at 512: its default candidate points evaluate far from the optimum, even though, as §4.2.2 shows, a top-K center still lies within a ±10∘ window of the optimum basin. That is, the pool covers the basin angularly but not as a low-vss point. Increasing the number of directions to 2048 lowers the point-oracle ratio to 1.91, partially reducing this point-metric limitation. However, this coarse-density increase does not monotonically improve the ranked top-K local result: the 512-direction pool with top-20 local verification already reaches a ratio of 1.58 for happy, while larger coarse pools can still rank the useful basin too low. By contrast, other meshes already have an oracle of 1.0–1.5 at 512, with no benefit from increasing the number of directions. Therefore happy is best treated as a hard case requiring a separate local-verification escalation branch, rather than as evidence that all meshes need denser global sampling or more feature weights. The happy-method of §4.2.2 implements this idea as an automatic escalation rule: it detects the low-confidence local-verification pattern and expands only the affected mesh, instead of increasing the global budget for every mesh.

#### Dimensional Normalization of the Objective Function.

The terms of the objective function J=R+P+B have different units (R,P∼ area​2, B∼ area). When the three terms are normalized to the same scale within the candidate pool and summed with equal weights, the mean Spearman correlation of raw J over the four non-happy meshes is 0.52 (the five-mesh average in Table 6 is 0.47); under min–max normalization it drops to 0.14  and to 0.10 under z-score normalization,  and the best-of-3 ratio also worsens from 2.82 to 3.53 and 3.86 respectively. This shows that the predictive power of raw J originates not from a balance of several terms but from the dominance of the build-plate term, which has the largest scale. Therefore the physical meaning of the weights is more properly secured not by simple dimensional normalization, but by removing non-predictive terms and reducing to a focus on R, B (§4.2.5).

## Verification against Four Implementations on a Controlled Five-Group Shape Set

While the preceding sections addressed the accuracy and generalization on stored Stanford mesh grids, this section directly contrasts SFTF’s algorithmic characteristics on a difficulty-controlled shape set with the four implementations TOMO_INT3, TOMO_CUDA, Python SFTF, and SFTF_C++. The verification was performed in batch with five_group_verification.py; SFTF generates a candidate pool from the input mesh alone (coarse 512 directions, candidate pool 1292), while TOMO_INT3 and TOMO_CUDA performed an exhaustive sweep on the same mesh over a yaw–pitch 3∘ grid (120×120=14,400 directions), critical angle 60∘, via DLL.  SFTF_C++ is an implementation that ports the same candidate-generation algorithm as Python SFTF to a C++ DLL, and the measured value is the pure computation time (sftf_compute_directions). The latest verification table (_5G_CA60deg_4Method_table.html) has the times of all four implementations filled in for all 35 meshes A–E.

### Experimental Setup

The five-group dataset (A–E) and the hardware/software environment used for all timing measurements are described in §3.1. For the 35 meshes A–E, we recorded the Python SFTF candidate-evaluation time, the TOMO_INT3 DLL sweep time, the TOMO_CUDA DLL sweep time, the SFTF_C++ candidate-evaluation time, the hit count and candidate score of the SFTF top direction, and the TOMO contour HTML. This section addresses the contrast of computational structure and implementation axes rather than the direction-accuracy ratio, and the quantitative analysis of direction accuracy is handled by the grid comparison of §4.2.3–§4.2.6.

### Overall Computation Time Comparison

Table [tab:fivegroup-timing] summarizes the four-implementation measurements of the 35 meshes. The most prominent characteristic is the contrast of fixed cost vs. adaptive cost. TOMO_INT3 takes about 4 seconds even for a cube with only 12 faces, having a large fixed cost almost independent of mesh complexity due to the nature of the yaw–pitch exhaustive sweep (because even on a 3∘ grid it evaluates all 14,400 directions). By contrast, the cost of SFTF adapts to mesh complexity, remaining 0.12–0.74 seconds for basic shapes, 1.0–3.2 seconds for organic benchmarks (C), 0.36–3.42 seconds for Thingi10K parts (D), and only 9.71–22.11 seconds even for large meshes of 500k–1M faces (E). Over all 35, Python SFTF is 1.4–34× faster than TOMO_INT3 (3∘), with an average speed ratio of about 9×. SFTF_C++ is additionally 3.9–233× faster than Python SFTF, and 10–5,160× faster than TOMO_INT3 (3∘). That is, SFTF is suitable not as a replacement for the expensive global sweep, but as a candidate generator that proposes promising orientations from several to several thousand times faster in front of it, and the denser the sweep grid is made to 1∘, the wider this gap becomes.

The result of TOMO_CUDA is more subtle. On the simple shapes of A/B, TOMO_CUDA is generally faster than TOMO_INT3 and shows the GPU-acceleration effect of the exhaustive sweep. However, on the organic and large meshes of C/D/E, TOMO_CUDA is often slower than TOMO_INT3, and on the overall average the TOMO_CUDA/TOMO_INT3 time ratio is about 1.07. This means that in this verification environment, the input preparation of the exhaustive sweep, the mesh complexity, and the DLL-internal data structures and memory-movement cost can govern performance more than the GPU backend itself. Therefore the core speed gain of this paper comes not from the “hardware axis of changing INT3 to CUDA” but from the “algorithmic axis of reducing 14,400 exhaustive directions on a 3∘ grid (129,600 at 1∘) to a 1292-candidate evaluation.”

### Group-wise Averages and Separation of Implementation Axes

Group-wise average execution time and speed ratio. The SFTF_C++ average is in ms, and the rest are in seconds. A TOMO_CUDA/TOMO_INT3 ratio greater than 1 indicates a case where the CUDA backend is slower than INT3.

| Group | py (s) | INT3 (s) | CUDA (s) | C++ (ms) | INT3/py | CUDA/INT3 |
| --- | --- | --- | --- | --- | --- | --- |
| A | 0.32 | 4.19 | 1.58 | 8.9 | 6–34× | 0.31–0.52 |
| B | 0.38 | 4.17 | 1.51 | 9.3 | 5–28× | 0.33–0.40 |
| C | 1.85 | 6.51 | 7.66 | 183.6 | 3–4× | 0.84–1.38 |
| D | 2.17 | 6.21 | 10.60 | 124.5 | 1.4–11× | 0.48–2.68 |
| E | 15.57 | 84.50 | 95.16 | 3123.5 | 1.5–8× | 1.02–1.45 |

### Scalability to Large Meshes (Group E)

Group E consists of 10 large Thingi10K meshes with face counts from about 5.1×105 to 1.04×106, examining whether SFTF works on high-resolution meshes beyond real-part scale. The key observation is the gentle increase of the SFTF cost. Even on E meshes whose face count is about 6–12× that of D6 (88,106 faces), Python SFTF candidate evaluation is 9.71–22.11 seconds, increasing only roughly sub-linearly with the face count. This is because the ray budget is fixed at an upper bound of 3000 via Kray=clip(0.02Nf,500,3000), so on large meshes the face preprocessing and BVH construction govern the cost while the ray casting cost saturates. SFTF_C++ finishes candidate evaluation on the same large meshes in 1.98–4.52 seconds, 3.9–6.9× faster than Python. These two candidate-generator costs are independent of the sweep grid (SFTF does not perform an exhaustive sweep).

By contrast, the 3∘ exhaustive sweep of TOMO_INT3 takes 22–133 seconds on group E (about 9×, 126–1,077 seconds, at 1∘). There are relatively short cases of about 22 seconds on identical 514,938-face meshes such as E8/E9, but on most large meshes tens of seconds to over a hundred seconds are needed. TOMO_CUDA was also 32–146 seconds, and was not faster than INT3 on large meshes in this verification environment. This shows that the algorithmic reduction of the number of candidates provides a more stable speed gain even on large meshes than the hardware acceleration of the exhaustive-sweep backend.

Internal consistency is also confirmed. E8 (64445) and E9 (64446) are essentially identical meshes (514,938 faces), and SFTF produced the same top score (−0.9824), the same direction, and the same hits (2,288), showing deterministic reproducibility, with the only difference being the execution time (15.05 vs. 14.93 seconds). Also, the number of valid face-to-face supports (hit) increases as the mesh detail becomes richer (e.g., E7 1,766, E8/E9 2,288), suggesting that the internal self-support structure of high-resolution shapes is faithfully reflected in SFTF’s flow tensor.

### Per-Direction Cost Structure Decomposition

Unlike TOMO_INT3, which summarizes each orientation with a single scalar vss, SFTF provides the cost of the top direction decomposed into the Rayleigh term R, the face-to-face support volume P, the build-plate penalty B, and the number of valid face-to-face supports hit (Table 15). This decomposition is a distinguishing characteristic of SFTF in that it directly provides an interpretation of “why that direction is good.”

Convex basic shapes such as cube, cylinder, and cone have hit=0, R=P=0, and the cost is composed entirely of the build-plate term B. This means the algorithm directly identifies that “there is no self-support relationship, and seating a plane on the floor makes support unnecessary.” Conversely, sphere, torus, and all organic/real parts have hit>0, so a face-to-face self-support structure exists, and R,P,B contribute together. For example, D10 (37416) has hit=496, P≈1.2×104, with face-to-face self-support dominant and B≈1.3, so the build-plate dependence is small, whereas D6 (37111) has B≈1.6×103, so the build-plate support requirement is large. The same contrast appears in organic meshes: dragon has hit=970 with R,P,B evenly distributed, whereas Bunny is dominated by B≈176. Such regime distinctions are not revealed by a single vss value alone.

Cost decomposition of the SFTF top direction of representative meshes. Convex basic shapes (cube, cone) are pure build-plate support with hit=0, R=P=0, and the rest show face-to-face self-support (P) and build-plate support (B) together.

| ID | Shape | hit | R​ | P​ | B​ |
| --- | --- | --- | --- | --- | --- |
| A1 | cube | 0 | 0 | 0 | 401.5 |
| A4 | cone | 0 | 0 | 0 | 179.8 |
| A2 | sphere | 498 | 47.3 | 109.5 | 0.02 |
| A5 | torus | 184 | 15.9 | 40.3 | 209.1 |
| B2 | hook | 235 | 19.9 | 55.4 | 128.7 |
| C1 | Bunny | 81 | 1.4 | 25.2 | 176.3 |
| C3 | dragon | 970 | 109.4 | 267.6 | 417.5 |
| D6 | 37111 | 386 | 227.9 | 932.8 | 1,613 |
| D10 | 37416 | 496 | 6,381 | 12,190 | 1.3 |

### Support Volume and Optimal Orientation Change with the Critical Angle

Even for the same shape, which faces are classified as overhangs changes with the printable critical angle θc, so the TOMO support volume and its global optimal orientation also depend on θc. To quantify this, we re-swept all 35 meshes over the five critical angles θc∈{0,30,45,60,90}∘ with TOMO_INT3 and TOMO_CUDA (a 3∘ grid). For each critical angle, we generated a table that gathers the four implementations on one page (_5G_CA⟨θc⟩deg_4Method_table.html) and a Polyscope overlay that captures the best/worst orientations of the five groups in one frame (_5G_CA⟨θc⟩deg_best_worst_polyscope.png, Figure 5).

Table 16 shows that the TOMO_INT3 global minimum vss of representative meshes decreases monotonically as θc increases. This is consistent with intuition: as θc grows, even steeper faces are allowed as self-support, so the support requirement decreases. The pattern of the decrease differs by shape. cube shows a threshold behavior: for θc≤30∘ the optimal orientation stands an edge up and vss>0, but for θc≥45∘ a plane is seated on the floor and vss=0; hook decreases smoothly from 1,736→0. For large/organic meshes, the global optimal orientation itself shifts; for example, the optimal (yaw,pitch) of 37009 jumps from near (≈​345∘,0∘) for θc≤45∘ to a different basin near (≈​150∘,186∘) for θc≥60∘.

TOMO_INT3 global minimum vss by critical angle (representative meshes, 3∘ grid). The support volume decreases monotonically as θc increases. In the region of large θc, almost all faces become self-supporting and support is essentially unnecessary, in which case the numerical residual of the MSST decomposition Vss=Vtc−Vo−Vnv can make vss appear as a small negative number (effectively 0).

| ID | Mesh | θc=0∘​ | 30∘​ | 45∘​ | 60∘​ | 90∘​ |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | cube | 1,769 | 1,769 | 0 | 0 | 0 |
| B2 | hook | 1,736 | 725 | 319 | 133 | 0 |
| C1 | Bunny | 49,780 | 23,660 | 13,970 | 5,147 | −7,857​ |
| C3 | dragon | 217,000 | 119,400 | 84,500 | 24,450 | −18,090​ |
| D1 | 37009 | 65,810 | 4,729 | 1,061 | −941​ | −7,226​ |
| D6 | 37111 | 99,200 | 58,980 | 38,740 | 9,703 | −6,270​ |
| E1 | 39507 | 26,380 | 12,510 | 6,493 | −6,337​ | −13,410​ |
| E7 | 61384 | 75,040 | 21,140 | 17,590 | −3,503​ | −14,610​ |

A contrast to note is that, while the TOMO optimal orientation shifts with θc above, the current SFTF candidates are invariant to θc. This is because the SFTF candidate generator is implemented with a setting that does not truncate the overhang coefficient Oi(n)=max(0,−mi⋅n) by the critical angle (USE_CRITICAL_ANGLE=False), and as a result the best/worst overlays of the five critical angles (Figure 5) are visually identical. Therefore Figure 5 shows the result of an arbitrary single critical angle (θc=60∘) as a representative, and θc-dependent candidate generation (SFTF with overhang truncation turned on) remains future work.

Best/worst build-orientation Polyscope overlay arranged by group for the five groups (A–E) (θc=60∘). A label is placed atop each group, with groups A–C in the top row and groups D/E (a vertical 5-row 2-column layout each) in the bottom row, grouped about the page center. Each mesh shows the original (gray), optimal orientation (blue), and worst orientation (red) side by side, displayed in the actual print pose by rotating the stored build direction to world +Z. Since SFTF candidates are invariant to θc, the overlays of the five critical angles are identical.

### Agreement of the Optimal Orientation

On trivial cases the two methods agree. For cube, cylinder, cone, u_bracket, c_clamp, hollow_box, and pipe_elbow, the TOMO_INT3 global optimal vss=0 (no support needed), and the SFTF top direction also selects an axis-aligned face-normal direction with a score nearly 0. That is, both methods arrive at the same conclusion that “seating a plane on the build plate makes support unnecessary.” On non-trivial organic/real parts (hook, groups C–D), the TOMO global optimum is located at an off-axis orientation with large vss, and SFTF likewise reflects the complex self-support structure with high hits and balanced R,P,B. However, this five-group experiment focuses on the contrast of computational characteristics and structural interpretation rather than the quantitative ratio of accuracy, and the quantitative analysis of direction accuracy is handled by the grid comparison of §4.2.3–§4.2.6.

### Discussion: Selection of the Ray Budget Kray

The ray budget in Eq. [eq:kray-budget] is a practical approximation parameter for the candidate generator, not a parameter of the final TOMO verification. For each build direction n, the source pool is first restricted to faces with positive overhang contribution AiOi(n)>0, and Kray faces are then selected by deterministic stratified sampling in proportion to this contribution. Thus, the sampling budget is spent on faces that are both geometrically large and strongly overhanging, rather than uniformly over the whole mesh.

The lower bound 500 prevents small meshes from being represented by only a handful of rays. For example, in the didactic FTree4x case (Nf=60), the formula gives Kray=500, but the weighted source pool has only Nsrc=8 faces; therefore all source faces are ray-cast and no subsampling occurs. The middle regime uses approximately 2% of the mesh faces, while the upper cap 3000 keeps the per-direction ray-casting cost bounded on large meshes. This cap is the reason why, in Group E, the SFTF cost grows gently even for meshes approaching 106 faces.

Figures 6–8 visualize this trade-off on Bunny 69k. The left panel shows the vertical-reference source pool and selected ray sources, while the right panel recomputes the SFTF candidate landscape using the same Kray value. For Bunny (Nf=69,662), the adaptive rule gives Kray=1,394. Although this is only 2.00% of all faces and 4.46% of the vertical source pool (Nsrc=31,247), the overhang-weighted sampling covers 11.91% of the vertical source-pool weight. The Kray=500 case is faster but visibly sparser, whereas Kray=3000 gives a denser estimate and can change the fine ordering of low-score basins. The low-score regions remain concentrated in a small number of candidate bands, which is sufficient for SFTF’s role as a warm-start candidate generator followed by TOMO/local verification. Therefore, the current clipped rule is justified as a cost–stability compromise: it avoids under-sampling small meshes, samples a meaningful overhang-weighted subset on medium meshes, and prevents large-mesh ray costs from growing without bound.

Bunny 69k source-face sampling and SFTF score landscape with fixed Kray=500. The right panel is recomputed with the same ray budget and uses the common color scale of Figures 7–8.

Bunny 69k source-face sampling and SFTF score landscape with the adaptive rule, Kray=clip(0.02Nf,500,3000)=1,394.

Bunny 69k source-face sampling and SFTF score landscape with fixed Kray=3000, corresponding to the upper cap used for large meshes.

In summary, this five-group verification quantitatively confirms the following SFTF characteristics. (i) Over all A–E, Python SFTF achieves a 1.4–34× computation-time reduction relative to TOMO_INT3 (3∘ sweep), and SFTF_C++ 10–5,160× (the gap grows on a denser 1∘ grid), (ii) a cost that adapts to mesh complexity (absence of fixed cost) and scalability up to ∼106-face meshes, (iii) a per-direction R/P/B/hit structure decomposition that a single vss cannot provide, (iv) agreement with the TOMO global optimum on trivial cases, (v) in the four-implementation comparison, that TOMO_CUDA is not always faster than INT3, and that the gain of the algorithmic axis of “reducing the evaluation targets” is more stable and larger than “changing the exhaustive-sweep backend,” (vi) that the C++ port enables candidate evaluation in ms units for small meshes and seconds for large meshes, (vii) that as the critical angle θc grows the TOMO support volume decreases monotonically and the global optimal orientation shifts, whereas the current SFTF candidates are invariant to θc (§4.3.6), and (viii) that the clipped Kray rule provides a bounded-cost ray budget while preserving the main low-score candidate bands (§4.3.8).

# Conclusion

This study proposed a direction evaluation method that combines the Support Flow Tensor Field and a build-plate penalty for build-orientation optimization in additive manufacturing. The eigenvector interpretation of the existing symmetric tensor is useful for shape directionality, but it does not sufficiently reflect direction-dependent overhangs, internal visibility, and the build-plate support requirement.

The current algorithm directly computes an asymmetric Support Flow Tensor for each candidate direction, and constructs a basic objective function J=R+P+B that combines the Rayleigh support cost, the face-to-face support volume, and the build-plate penalty. In the actual final candidate selection, it applies a rank-normalized tuning score T of J,R,P,B,H,S,σ1 to a candidate pool made by coarse spherical sampling and small-angle refined sampling, to choose promising orientations to pass to slicer/TOMO verification. When used this way, verifying the ±10∘ neighborhoods of only the top-20 candidates recovers a mean support-volume ratio of 1.13 (maximum 1.58) while inspecting 3.57% of the 1∘ grid, and the happy-method automatically widens this budget only for detected hard cases (§4.2.2).

The quantitative analysis (§4.2.3–§4.2.6) shows both the strengths and limitations of the method. Cross-validated SFTF is consistently superior to PCA and Support Tensor eigenvector directions, supporting the starting assumption. At the same time, most of the predictive power originates from the build-plate penalty and the internal visibility evaluation, the marginal contribution of some tensor-derived terms such as nuclear/σ1 is small, and the calibrated rank tuning weights show an optimistic bias on unseen meshes. Meanwhile, in §2.4.1–§4.2.4 we showed that the build-plate penalty is exactly equivalent to the Rayleigh contribution obtained by adding a virtual ground node to the flow tensor. This unifies the term driving the predictive power into the tensor contraction R=R+B, justifying the “tensor” formulation, and confirms that R, excluding the negative-signal face-to-face support volume P, is consistently superior to the existing J.

Therefore, in future work we will (i) determine the weights on a cross-validation basis rather than a calibration set to control overfitting, (ii) remove terms with marginal contribution or replace them with features highly correlated with the actual vss, such as face-to-face self-support coverage or a support-volume proxy, (iii) normalize the dimensions of the objective-function terms to secure the physical meaning and transferability of the weights, (iv) add projection-area, print-height, and contact-stability terms to extend to a multi-objective orientation optimization model, and (v) as observed in §4.3.6, since the TOMO optimal orientation shifts with the critical angle θc while the current SFTF candidates are invariant to θc, extend to θc-dependent candidate generation with overhang truncation turned on. For cases where the ordinary top-K candidate generation does not include a stable near-optimal basin, the happy-method provides an automatic escalation path, and future work will refine its thresholds on a larger family-held-out validation set.

# Statements and Declarations

## Funding

Funding information is provided separately in the submission system and title-page file to preserve double-anonymous review.

## Competing interests

The author declares no competing interests.

## Data availability

The numerical results reported in this manuscript are based on cached mesh-evaluation records, support-volume grids, and generated summary tables maintained with the research code. An anonymized repository or supplementary archive can be supplied during peer review, and the permanent public repository link can be provided after acceptance. Third-party mesh models should be obtained from their original sources subject to their respective licenses.

## Code availability

The code and scripts supporting the reported results will be made publicly available upon acceptance. An anonymized review archive can be provided during peer review if required.

## Use of artificial intelligence tools

AI coding assistants, including OpenAI Codex and Anthropic Claude Code, were used to assist with portions of research-code implementation, figure-generation scripts, and manuscript-support automation. All generated code, experimental results, and manuscript changes were reviewed and validated by the author.

## Ethics approval and consent to participate

Not applicable.

## Consent for publication

Not applicable.

# TOMO_INT3 vss Contour Landscapes

This appendix overlays the optimal/worst orientations of INT3 and SFTF onto the stored TOMO_INT3 vss grid used as the comparison baseline in the main text (Experimental/etc/*_tomo_int3_vss_grid_1deg_60deg.npz), to directly compare where the predictions of the two methods lie on the exhaustive-search landscape. Each grid is the exhaustive-sweep vss computed at yaw–pitch 1∘ resolution and a critical angle of 60∘, and the entire 361×361 grid is displayed as a filled contour of the RdBu_r diverging colormap (the bluer, the smaller vss and the better orientation; the redder, the larger support volume; the colormap is the same as the HTML figures in the main text). Four kinds of markers are overlaid, 3 each: stars (⋆, INT3 optimal; the three different basins with the smallest vss on the grid), X (INT3 worst; the three basins with the largest vss), circles (SFTF optimal; the top three directions of SFTF_C++ sftf_cpp_optimal), and diamonds (SFTF worst; the bottom three directions sftf_cpp_worst), with the SFTF directions placed at their nearest grid cell. The legend is placed below each figure so as not to obscure it. Therefore, the closer SFTF optimal (circles) is to INT3 optimal (stars) and SFTF worst (diamonds) to INT3 worst (X), the better the thousands-of-times-cheaper SFTF reproduces the good/bad orientations of the exhaustive search. These static figures were rendered from the stored npz grids and the verification records (Group_C*.json) with scripts/render_appendix_contours.py.

Comparison of INT3/SFTF optimal/worst orientations on the TOMO_INT3 vss landscape of Bunny 69k (blue is good). The markers are 3 each of INT3/SFTF optimal/worst, with kinds following the legend below the figure. The global optimum is vss=3201 (yaw=265∘, pitch=49∘; Table 1 in the main text), and lies not at a single point but within a broad low-cost basin.

Comparison of INT3/SFTF optimal/worst orientations on the TOMO_INT3 vss landscape of manikin (blue is good). The markers are 3 each of INT3/SFTF optimal/worst, with kinds following the legend below the figure. The global optimum is vss=4658 (yaw=193∘, pitch=7∘), and befitting a vertically elongated shape, the low-cost region is distributed in a narrow pitch band across the full yaw range.

Comparison of INT3/SFTF optimal/worst orientations on the TOMO_INT3 vss landscape of dragon 100k (blue is good). The markers are 3 each of INT3/SFTF optimal/worst, with kinds following the legend below the figure. The global optimum is vss=24447 (yaw=162∘, pitch=258∘), and the absolute vss scale is large due to the abundant surface detail, but the global optimum forms a relatively distinct basin.

happy 50k before and after happy-method escalation on the same TOMO_INT3 vss landscape. Without escalation, the base top-10 local verification selects a high-cost basin (vss=17,991, ratio 8.52, 2.20% grid budget). With the happy-method trigger, the search is expanded to the adaptive final set and recovers the TOMO optimum (vss=2,112, ratio 1.00, 7.48% grid budget). This visualizes why happy is treated as a local-verification hard case rather than as a score-weight tuning failure.

Comparison of INT3/SFTF optimal/worst orientations on the TOMO_INT3 vss landscape of lucy 50k (blue is good). The markers are 3 each of INT3/SFTF optimal/worst, with kinds following the legend below the figure. The global optimum is vss=5036 (yaw=182∘, pitch=359∘), and being located near the pitch boundary (359∘), it requires basin search that considers the yaw–pitch periodicity.

These five landscapes visually support the observations of the main text. When the global optimum lies within a broad basin, as in Bunny and dragon, a near-optimal orientation is easily included in the candidate pool with coarse spherical sampling alone, whereas when the low-cost region is narrow and scattered across several places, as in happy, the ranking-only point metric becomes unreliable and local windowed verification is needed. Also, when the optimum is located at a yaw–pitch boundary, as in lucy, it shows the necessity of a basin representation that considers the periodicity emphasized in the main text.

1. 	Gibson I, Rosen D, Stucker B (2015) Additive manufacturing technologies, 2nd ed. Springer, New York

2. 	Das P, Mhapsekar K, Chowdhury S, et al (2017) Selection of build orientation for optimal support structures and minimum part errors in additive manufacturing. Computer-Aided Design and Applications 14:1–13. https://doi.org/10.1080/16864360.2017.1308074

3. 	Wang CCL, Leung Y-S, Chen Y (2010) Solid modeling of polyhedral objects by layered depth-normal images on the GPU. Computer-Aided Design 42:535–544. https://doi.org/10.1016/j.cad.2010.02.001

4. 	Vaissier B, Pernot J-P, Chougrani L, Véron P (2019) Genetic-algorithm based framework for lattice support structure optimization in additive manufacturing. Computer-Aided Design 110:11–23. https://doi.org/10.1016/j.cad.2018.12.007

5. 	Jiang J, Stringer J, Xu X, Zhong RY (2018) Investigation of printable threshold overhang angle in extrusion-based additive manufacturing for reducing support waste. International Journal of Computer Integrated Manufacturing 31:961–969. https://doi.org/10.1080/0951192X.2018.1466398

6. 	Mirzendehdel AM, Suresh K (2016) Support structure constrained topology optimization for additive manufacturing. Computer-Aided Design 81:1–13. https://doi.org/10.1016/j.cad.2016.07.006

7. 	Langelaar M (2018) Combined optimization of part topology, support structure layout and build orientation for additive manufacturing. Structural and Multidisciplinary Optimization 57:1985–2004. https://doi.org/10.1007/s00158-017-1877-z

8. 	Dai C, Wang CCL, Wu C, et al (2018) Support-free volume printing by multi-axis motion. ACM Transactions on Graphics 37:1–14. https://doi.org/10.1145/3197517.3201342

9. 	Gaynor AT, Meisel NA, Williams CB, Guest JK (2014) Topology optimization for additive manufacturing: Considering maximum overhang constraint. In: 15th AIAA/ISSMO multidisciplinary analysis and optimization conference. p 2036

10. 	Garcke H, Lam KF, Nürnberg R, Signori A (2023) Overhang penalization in additive manufacturing via phase field structural topology optimization with anisotropic energies. Applied Mathematics & Optimization 87:44. https://doi.org/10.1007/s00245-022-09939-z

11. 	Shin S, Park Y, Yoon GH (2025) Topology optimization for multi-axis additive manufacturing considering overhang and anisotropy. International Journal of Mechanical Sciences 301:110443. https://doi.org/10.1016/j.ijmecsci.2025.110443

12. 	Smith R, Hollenbeck MC (2025) Build orientation for additive manufacturing of complex structures

13. 	Ezair B, Massarwi F, Elber G (2015) Orientation analysis of 3D objects toward minimal support volume in 3D-printing. Computers & Graphics 51:117–124. https://doi.org/10.1016/j.cag.2015.05.009

14. 	Jung JY, Chee S, Sul IH (2022) Support structure tomography using per-pixel signed shadow casting in human manikin 3D printing. Fashion and Textiles 9:1–18. https://doi.org/10.1186/s40691-022-00290-z

15. 	Kim JR, Sul IH (2026) Fast prediction of 3D printing optimal orientation using general-purpose graphic processor unit calculation. 3D Printing and Additive Manufacturing 13:50–62. https://doi.org/10.1089/3dp.2024.0165

16. 	Kanatani K-I (1984) Distribution of directional data and fabric tensors. International Journal of Engineering Science 22:149–164. https://doi.org/10.1016/0020-7225(84)90090-9

17. 	Horn RA, Johnson CR (2013) Matrix analysis, 2nd ed. Cambridge University Press, Cambridge

18. 	Golub GH, Van Loan CF (2013) Matrix computations, 4th ed. Johns Hopkins University Press, Baltimore

19. 	Roth SD (1982) Ray casting for modeling solids. Computer Graphics and Image Processing 18:109–144. https://doi.org/10.1016/0146-664X(82)90169-1

20. 	Cochran WG (1977) Sampling techniques, 3rd ed. Wiley, New York

21. 	Spearman C (1904) The proof and measurement of association between two things. The American Journal of Psychology 15:72–101. https://doi.org/10.2307/1412159

22. 	Saff EB, Kuijlaars ABJ (1997) Distributing many points on a sphere. The Mathematical Intelligencer 19:5–11. https://doi.org/10.1007/BF03024331

23. 	González Á (2010) Measurement of areas on a sphere using Fibonacci and latitude–longitude lattices. Mathematical Geosciences 42:49–64. https://doi.org/10.1007/s11004-009-9257-x

24. 	Möller T, Trumbore B (1997) Fast, minimum storage ray-triangle intersection. Journal of Graphics Tools 2:21–28. https://doi.org/10.1080/10867651.1997.10487468

25. 	Zhou Q, Jacobson A (2016) Thingi10K: A dataset of 10,000 3D-printing models. arXiv preprint arXiv:160504797

26. 	Stone M (1974) Cross-validatory choice and assessment of statistical predictions. Journal of the Royal Statistical Society: Series B (Methodological) 36:111–133. https://doi.org/10.1111/j.2517-6161.1974.tb00994.x

27. 	Hoerl AE, Kennard RW (1970) Ridge regression: Biased estimation for nonorthogonal problems. Technometrics 12:55–67. https://doi.org/10.1080/00401706.1970.10488634

28. 	Han J, Kamber M, Pei J (2011) Data mining: Concepts and techniques, 3rd ed. Morgan Kaufmann, Waltham, MA

Previous MSST [14] and GPU-accelerated (GPU-MSST) [15] work. Public source code: https://github.com/cfms-lab/Tomo_GPU2024.

The symmetric tensor i​AimimiT is the (area-weighted) outer-product sum of the normals mi, a classical shape descriptor of the structure tensor / orientation tensor family. See: [16].

A symmetric matrix such that xTMx≥0 for all vectors x. This means all eigenvalues are ≥0. See: [17]

A direction x satisfying Mx=λx, representing the axes that the matrix “stretches” most strongly. See: [18]

A technique that shoots a ray to find the first surface it encounters. Here it is used to shoot a ray from an overhang face in the direction opposite the build direction (−n) to find the receiver face below. See: [19]

Stratified sampling. A method that divides the whole into several intervals (strata) and then draws samples from each stratum, obtaining a more evenly representative sample than simple random sampling (stratified sampling). See: [20]

Rayleigh quotient: for a symmetric matrix A and vector x, the scalar defined as xTAxxTx. Since ∥n∥=1 here, nTFs(n) n itself is the Rayleigh quotient at direction n, and we flip its sign to interpret it as a “support cost.” (A quantity closely related to eigenvalues; [17])

Singular value and singular value decomposition (SVD): the non-negative scale factors that appear when decomposing an arbitrary matrix into rotation–scale–rotation, representing how much the matrix “stretches” along each axis. For the non-symmetric F(n), singular values are used instead of eigenvalues. See: [18]

Nuclear norm: the sum of singular values k​σk. A type of Schatten norm, a measure of the overall “size” of a matrix (a type of matrix norm). See: [17]

In this paper, “rank tuning” is not a standard term but refers to the post-processing procedure of this study. Each feature is normalized to its rank within the candidate set (0 ∼ 1) (see the formula in §4.2.5), and then linear weights are learned on these rank-normalized features to re-rank the candidates. For the concepts of rank and rank correlation, see [21].

A method that scatters points nearly uniformly on a sphere using the golden angle (≈137.5∘), commonly used to obtain evenly distributed direction samples (the 137.508∘ rotation in the code is this). For classical results on quasi-uniform distributions of points on a sphere, see Saff and Kuijlaars [22]; for the spherical application of the Fibonacci lattice and its uniformity analysis, see González [23].

Non-maximum suppression (NMS): a technique that adopts candidates from the best-scoring one in turn, but removes candidates too close to an already-adopted one to eliminate duplicates (originating from object detection). Here “closeness” is defined as the angle between two orientations, to prevent neighboring directions of the same basin from being redundantly selected.

Spearman rank correlation coefficient: a correlation computed after converting two variables into ranks instead of values, where +1 means a perfectly monotonically increasing relationship and −1 a perfectly monotonically decreasing one. It measures whether “large values go together and small values go together” even if the relationship is not linear. See: [21]

Cross-validation: a method that sets aside part of the data from training and uses it only for validation, to estimate performance on unseen data. LOMO is a “leave one mesh out” scheme that excludes one mesh, fits the weights on the rest, and then measures performance on the excluded mesh. See: [26]

Ridge regression: linear regression that adds an L2 penalty so that the regression weights do not become too large, reducing overfitting. See: [27]

Feature scaling (normalization) that linearly maps the minimum to 0 and the maximum to 1. See: [28]

Standard score. A normalization that subtracts the mean and divides by the standard deviation to make “mean 0, variance 1.” See: [28]

To handle a large-scale batch that sweeps 35 meshes over 5 critical angles (θc∈{0,30,45,60,90}∘) with two backends, the sweep grid of this verification was set to a coarser 3∘ than the prior GPU-MSST’s 1∘ (360×360=129,600 directions). On a denser 1∘ grid, the TOMO sweep time increases about 9×, widening the speed gap relative to SFTF further.